## Types of Springs: Unravelled

A mechanical spring is a rigid wire material drawn into a coil shape designed to deform elastically under a mechanical load. In this way, the spring stores mechanical energy, releasing it when unloaded to achieve the intended mechanical functionality. Each spring is uniquely designed to a specific form and function, resulting in a marketplace full of different product shapes, sizes, and materials.

The spring is a simple yet clever design that can store mechanical energy in small and large engineering and industrial applications. Indeed, springs are used everywhere, from car suspensions to clothespins. Their usefulness is impossible to understate.

There are dozens of types of springs on the market today, and it is our goal to provide you with a map of the marketplace by using a few distinctive features of form, fit, and function as guideposts.

## Spring Types: By Application

The design application directly determines the direction and type of force that the spring must absorb. In essence, springs are designed to either resist tension loads (extension springs), compressive loads (compression springs), or torsional loads (torsion springs).

- Compression Springs are designed to resist compressive forces, meaning the coils are designed to be pressed together. They store compressive energy mechanical energy when compressed.

- Extension Springs are designed to resist tension forces. These springs are similar to compression springs, except their coils are wound tighter because they are designed to be extended during operation. Extension springs absorb energy as the coils separate.

- Torsion Springs are engineered to resist twisting forces, as the name suggests. They are tightly wound wire coils that unwind elastically as they absorb torsional loads.

## Spring Types: By Shape

The shape of the spring is also another critical differentiating feature among springs.

- Helical Springs look like a coiled wire or spiral. These springs are categorized by their designed loading application:

  - Compression Coil Springs are cylindrical coil-shaped springs designed to compress when loaded.

  - Extension Coil Springs are cylindrical coil-shaped springs designed to extend when loaded.

  - Torsion Coil Springs are tightly wound coils designed to resist tension loads

- Tapered Springs are conical-shaped thanks to the tapered twist of the coil at the top of the spring. This shape provides extra lateral stability in the spring under large compression forces.

- Disc Springs look like stacked conical washers and function as compressive springs in applications with limited installation space.

- Leaf Springs are large, arc-shaped springs comprised of stacked, rectangular, staggered-length steel pieces. They are primarily used in automotive suspensions.

- Volute Springs are conical shaped, similar to tapered springs, except the wire is rectangular with a large vertical height dimension.

- Barrel Springs coils feature large middle sections, giving them a barrel-shaped appearance, allowing for increased stability.

- Flat Springs are typically slightly curved conical washers that deform under load. They are used in limited-space applications.

- Spiral Springs are specialized torsion springs fabricated by coiling a flat, rectangular wire into a spiral configuration. They are used in specialized components such as wristwatches.

- Wave Springs are compression springs that are far more compact than ordinary compression springs due to their interwoven coil design.

- Garter Springs can be considered a typical compression spring shaped into a circular doughnut shape. This shape makes them useful in shaft seal designs.

## Spring Types: By Material

Metallic Springs

Steel Springs

- Carbon steel springs are an economical choice for applications with moderate loading requirements.

- Stainless Steel springs are commonly found in machine applications that require corrosion resistance.

- Alloy Steel springs are usually specified in high-load or high-temperature applications.

- High-carbon steel is a specialty steel that is highly ductile and can resist very high loads. This allows the springs to be wound tightly and have a small profile.

Non-Metallic Springs

Plastic Springs are used in highly corrosive environments such as food processing and are only suitable for low-load applications.

Rubber Springs are possible because of rubber’s elastic properties, so no coil is needed. They are used in specialized suspension systems.

Specialty Material Springs

Ceramic Springs offer high heat and corrosion resistance and are used in environments such as semiconductor manufacturing equipment.

Glass Springs are highly specialized springs historically used in laboratory chemistry operations.

Titanium Springs have a high strength-to-weight ratio, making them ideal for aerospace applications.

## Modern Spring Manufacturing

Spring manufacturing has experienced a variety of innovations that now allow for the quick fabrication of custom spring designs, which leads to significantly reduced manufacturing lead times on custom spring designs.

Specifically, CAD and CAM tools have enabled this productivity increase. These innovations allow CAD spring designs to be quickly created on a computer and transferred directly to a CNC spring coiling machine for production runs. Furthermore, automated production machines and robotics can perform complicated multi-axis manufacturing techniques such as spring coiling.

This functionality is critical for industries that require large volumes of customized spring designs. With custom design solutions, springs can be quickly manufactured using exotic materials and geometries to meet customer design requirements.

As a manufacturer that leads the industry in customized solutions, Century Spring is proud to offer customized manufacturing solutions for compression springs. Let Century Spring partner with you and deliver high volumes of customized springs at short product lead times!

## Explore Century Spring’s Extensive Spring Offerings and In-House Expertise Today

Century Spring specializes in both stock and custom springs as a quality-first manufacturer.  It offers rapid turnaround, shipping, and delivery on over 40,000 in-stock designs available to ship today!

Furthermore, Century Spring now offers customized spring design and fabrication services on specialized designs. Century Spring’s team of highly skilled engineers is available today to help you design a customized spring solution to meet your application's unique challenge while minimizing total development time so you can tackle your next pressing challenge. Don’t waste any more of your valuable design time today; let the spring design experts take the lead!

Because with Century Springs, you always get the spring you need precisely when needed. Century Spring is ISO 9001 certified and all parts are always Made in the USA.

Shop our Springs and Request a Quote Today!